|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION

SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| M*95 Product Number: 7115; 7495 | Janitorial |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Corrosive liquids, n.o.s., 8, UN 1760, III, (contains hydrochloric (muriatic) acid) |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 3 - Serious | Health: | 3 - Serious; Chronic Hazard |
| Fire: | 0 - Minimal | Fire: | 0 - Minimal |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | See Section VIII |

SECTION II: HAZARDOUS INGREDIENTS

SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | NTP, IARC or OSHA Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| *Hydrochloric acid | 5-10 | 7647-01-0 | Not Established | Not Established | 7 (3 ACGIH) | No |

Comment: *This substance is subject to the reporting requirements of Section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 (SARA) and of 40 CFR 372.

SECTION III: PHYSICAL DATA

SECTION III: PHYSICAL DATA

| Boiling Point: | 210-212 °F | Vapor Pressure: | Undetermined |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Complete |
| pH: | < 1 | Specific Gravity (H2O=1): | 1.04 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | < 1-2 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Opaque green liquid, pine fragrance |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | > 212°F | Method Used: | ASTM-D56 |
| --- | --- | --- | --- |
| Flammable Limits: | Not Established | Flame Extension: | N/A |

| Extinguishing Media: | Product does not support combustion. Use extinguishing media appropriate for surrounding fire. |
| --- | --- |
| Special Fire Fighting Procedures: | Wear NIOSH approved self-contained breathing apparatus and protective clothing. Cool fire-exposed containers with water spray. |
| Unusual Fire & Explosive Hazards: | Combustion products are toxic. Acid action on most metals may release hydrogen, an explosive gas. |

SECTION V: HEALTH HAZARD DATA

SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not Established | Primary Routes of Entry: | Inhalation, Skin Contact, Eyes & Oral |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | CORROSIVE. CAUSES IRREVERSIBLE EYE DAMAGE AND SKIN BURNS: Symptoms may include pain, redness, swelling and tissue damage. Harmful if absorbed through the skin. MAY BE FATAL IF SWALLOWED. Symptoms may include nausea, vomiting, pain and diarrhea. INHALATION OF VAPORS OR MIST MAY CAUSE IRRITATION OR HARM TO THE RESPIRATORY TRACT. Breathing vapors or mist may cause coughing and difficulty breathing. Do not get in eyes on skin or clothing. Do not swallow. Avoid breathing product vapors or mist. Wash thoroughly with soap and water after handling. |
| --- | --- |
| Conditions Aggravated by Use: | Use of this product may aggravate preexisting skin; eye and respiratory disorders including asthma and dermatitis. |

Emergency & First Aid Procedures:

| Eyes: | Flush eyes with water for at least 15 minutes. Remove contact lenses. Get medical attention. |
| --- | --- |
| Skin: | Remove contaminated clothing. Rinse skin immediately with plenty of water for at least 15 minutes. Get medical attention if irritation persists. Wash contaminated clothing before reuse. |
| Ingestion: | Get immediate medical attention. Do not induce vomiting. Drink one or two glasses of water to dilute product. Do not give anything by mouth to an unconscious person. |
| Inhalation: | Move person to fresh air. Get medical attention if irritation persists. |

NOTE TO PHYSICIAN: Probable mucosal damage may contraindicate the use of gastric lavage.

SECTION VI: REACTIVITY DATA

SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | Strong alkalis; hypochlorites (bleach) ammonia; cyanides. |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | HCL, CO, CO2 H | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES

SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | Small spills of one quart or less may be flushed with plenty of water to sanitary sewer system (If permitted by local sewer regulations). Dike and contain larger spills with inert material and transfer liquid to containers for recovery or disposal. |
| --- | --- |
| Waste Disposal Method: | Dispose of in compliance with all federal, state and local laws and regulations. |

SECTION VIII: SPECIAL PROTECTION INFORMATION

SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | Not normally required when good general ventilation is provided. However, if exposure limits (see Section II) are exceeded or respiratory irritation occurs, use of a NIOSH approved respirator suitable for the use-conditions and chemicals in Section II should be considered. |
| --- | --- |
| Ventilation: | Provide good general ventilation. Local exhaust ventilation may be necessary for some operations. |
| Protective Gloves(Specify Type): | Rubber or neoprene gloves are recommended. |
| Eye Protection(Specify Type): | Splash goggles are recommended to prevent eye contact. |
| Other Protective Equipment: | Acid resistant aprons and boots are recommended where body or foot contact is likely. Eye wash stations and washing facilities should be readily accessible. See 29 CFR 1910.132-138. |

SECTION IX: SPECIAL PRECAUTIONS

SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Do not contaminate water, food or feed by storage or disposal. Keep from freezing. Do not use with other chemicals. Mixtures of this product with other chemicals, particularly those containing chlorine bleach or ammonia may result in the production of toxic gases, fire or explosion. |
| --- | --- |
| Other Precautions: | Keep out of reach of children. |

| © SCC 06/01/2007 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| M*95 | Effective Date: | 06/01/2007 | Supercedes: | 06/30/2005 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | Update Section II |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.